Addressing Modes

Addressing modes in assembly language specify how the operand of an instruction is determined. Different modes allow flexibility in how data is accessed or manipulated. There are four registers in iAPX88 architecture that can hold address of data, and they are BX, BP, SI, and DI.

1. Immediate Addressing Mode

- Syntax: MOV AX, 5

- The operand is a constant or immediate value.

- Example: MOV AX, 0x1234 moves the immediate value 0x1234 directly into the AX register.

- Usage: For loading constants into registers.

2. Register Addressing Mode

- Syntax: MOV AX, BX

- The operand is a register, and data is transferred between registers.

- Example: MOV AX, BX moves the content of register BX into AX.

- Usage: For operations involving registers directly.

3. Direct Addressing Mode

- Syntax: MOV AX, [1234H] or MOV AX, [num1]

- The source operand is a memory address, and the instruction accesses data stored at that address.

- Example: MOV AX, [1234H] moves the value stored at memory address 1234H into AX.

- Usage: For reading or writing specific memory locations.

4. Direct Addressing with Displacement (Offset) Addressing Mode

- Syntax: MOV AX, [num+2]

- num is a label or constant that represents a memory address.

- num + 2 means that the processor will access the memory location starting at num, but with a displacement of 2 bytes added to the base address.

- The data at this new address (num + 2) will be moved into AX.

5. Based Register Indirect Addressing Mode

- Syntax: MOV AX, [BX] or MOV [BP], AL

- The memory address is specified indirectly through a register. BX is allowed to hold an address but BL or BH are not. Address is 16bit and must be contained in a 16bit register.

- Example: MOV AX, [BX] moves the value stored at the memory address in BX into AX.

- Usage: Allows for dynamic memory access where the address is stored in a register.

- We used the BX register for it and the B in BX and BP stands for base therefore we call register indirect memory access using BX or BP, “based addressing.”

- When SI or DI is used we name the method “indexed addressing.”

6. Based Register Indirect + Offset Mode

- Syntax: ADD AX, [BX+2]

- Base Register: BX holds a memory address.

- Offset/Displacement: +2 is the offset added to the contents of BX.

- Effective Address: The sum of BX and the offset 2 forms the final address where the operand is located.

- The value stored at this effective address is then added to the AX register.